W-16

YEAR 12 PHYSICS

ASSIGNMENT 4 - ELECTROMAGNETISM

Name:  											Mark:

1.	Drawn below are diagrams representing three fields. Use the information contained in the 	blanks in the following sentences. Any field diagram can be used more than once.

(5 marks)

Diagram 			 could represent the gravitational field of a mass.

Diagram 			 could represent the electric field around a positively charged particle.

Diagram 			 could represent the electric field of a negatively charged particle.

Diagram 			 represents a current carrying wire directed  				 the 	page.

2.	An electron moving with an initial velocity u has initial kinetic energy EKi . It enters a uniform 	electric field with field strength E, as shown in the diagram below. The electron’s final kinetic 	energy EKf is equal to 4EKi.

The electric field strength is now doubled. If another electron, having initial kinetic energy EKi 	enters the field, determine this electron’s final kinetic energy in terms of EKi.

You should ignore the effects of gravity. 								(4 marks)

3.	Draw the resultant electric field with at least 5 lines for each of the following situations.

(4 marks)

4.	An ammeter is a device that is used to determine the magnitude of an electric current. The 	unknown current is passed through a coil of wire in a magnetic field. The turning effect of the 	current-carrying coil is balanced by a spring and a corresponding value is read from the 	meter.

(a) 	Use the photograph below of an ammeter’s scale to determine the magnitude of the 		current passing through it, as well as the absolute and relative uncertainty for this 		value. 												(3 marks)

Current: 			mA

Absolute uncertainty:

Relative uncertainty:

(b)	A simplified diagram representing one current-carrying wire of the ammeter’s coil 			between two magnets is shown below. Draw at least five field lines to show the 			resultant magnetic field between the magnets. 					(4 marks)

(c) 	Calculate the magnitude of the force, in newtons, acting on the wire carrying a current 		of 1.45 A in the simplified diagram on page 3, given that the magnetic field strength 		is 4.25 × 10-2 T and the length of the wire in the field is 2.50 × 10-2 m. 	(3 marks)

(d) 	The actual ammeter shown has 250 turns of wire that form a square coil with sides of 		3.20 × 10-2 m. Determine the magnitude of the current in amperes, given that the 		spring provides a restoring torque of 2.65 × 10-2 N m in the magnetic field strength of 		4.25 × 10-2 T. 											(4 marks)

5.	Jake wanted to determine the strength of a magnetic field by conducting an investigation. In 	this investigation, two identical cylindrical permanent magnets, each 2.0 cm in diameter, 	were placed opposite each other on either side of an aluminium channel. A current was 	passed along a 20 cm copper rod, which in turn was placed perpendicularly in the magnetic 	field. The interaction between the permanent magnets and the current-carrying wire 	produced a downward force acting on the magnets, which was measured using a digital 	balance. Photographs of the equipment are shown below, as is a schematic diagram of the 	circuit.

(a) 	Using the photograph above, for magnets labelled Q and R, write either ‘North’ or 		‘South’ in the space below to indicate which pole the magnet would need to have next 		to the channel to provide the magnets with a force directed downward (into the pan of 		the balance).  											(2 marks)

For magnet Q, the 		 pole would be next to the channel.

For magnet R, the 		 pole would be next to the channel.

(b) 	A table of results for this investigation is shown below.

(i)  	Complete the last column in the table above with values expressed to two 				significant figures. 									(2 marks)

(ii)  	Use the data from the table to plot a straight-line graph on the grid provided, 			demonstrating the relationship between the current and force. 		(4 marks)

(iii)  	Use your graph to determine the force that should be measured when a current 			of 4.0 A flows through the copper rod. Express your answer using appropriate 			significant figures. 									(3 marks)

(iv)  	Determine the gradient of your line of best fit. Include units in your answer.  (3 marks)

(v)  	Use your gradient to determine the experimental value of the magnetic field strength. 		Include units in your answer. Show all workings. 					(4 marks)

6.	An electron gun is a very important component of many devices, including particle 	accelerators, electron microscopes and cathode-ray tubes. A schematic diagram of an 	electron gun is shown below.

Assume the average initial velocity of a thermal electron is zero. The anode voltages are 	V1= 1500 V and V2 = 4500 V and the distances between the cathode and anodes are as 	shown above.

(a) 	Calculate the velocity in m s-1 of the thermal electrons as they pass through the first 		anode. 												(4 marks)

(b) 	Calculate the average acceleration in m s-2 of an electron in the region between the 		cathode and the first anode. 									(3 marks)

(c) 	Complete the sketches that qualitatively represent the situation on the axes below. The 		first graph, of distance versus potential difference, has been completed for you. 																(6 marks)

(d) 	Calculate the electrical work done by the electric field in moving one electron from the 		first anode to the second anode. Include units with your answer. 		(3 marks)

7.	An electron moving at 0.900c enters a region of space and follows a path that has a constant 	radius of 0.348 m while in the magnetic field shown on the diagram, before striking a target 	anode.

(a)  	Draw the magnetic field enclosed in the indicated space. 				(2 marks)

(b)  	(i) 	Derive the formula  . 							 (3 marks)

(ii)	Use this formula to calculate the field strength needed to direct an electron along 			this path. Include units in your answer. 						 (4 marks)

(iii) 	Describe how each of the changes below affect the charged particle’s path in the 			magnetic field. 										(4 marks)

8.	In the diagram below, the arrow represents a stream of electrons, moving with velocity v, 	entering a solid copper strip. The electrons are moving in the direction M to N. A magnetic 	field of strength B, perpendicular to the strip is switched on.

(a) 	Explain why electrons will begin to collect on the right hand edge of the strip and why 		an electric field develops across the strip. Express the voltage (V) due to the electric 		field in terms of the electric field strength (E) and the distance across the strip (d).

(4 marks)

The phenomenon of a voltage being produced across a current carrying conductor due to the 	presence of a magnetic field is called the Hall effect, and the voltage is termed the Hall 	voltage. It is utilised in probes used to measure magnetic field strength.

(b) 	For the probe in the diagram below, draw an arrow to indicate the direction of the 			electric field in the strip. 									(1 mark)

(c) 	The Hall voltage can be calculated using the equation

where 	I = electric current

B = magnetic field strength

t = thickness of the strip

n = number of electrons per m3

e = charge on an electron

Calculate the magnetic field strength when V = 2.25 mV, I = 1.80 A, t = 1.25 × 10-4 m 		and n = 1.52 × 1025 m-3. 									(3 marks)

(d) 	Calculate the magnetic force exerted on the electrons if they are moving with velocity 		1.17 m s−1. 											(3 marks)