|  | Year 12 ATAR Physics Unit 3 2018 Test 3 Moments and Equilibrium 3.0% NAME: …………………………………………………. Data: See Data Sheet Approx. marks shown. (55 marks) |
| --- | --- |

When calculating numerical answers, show your working or reasoning clearly. Give final answers to three significant figures and include appropriate units where applicable.

When estimating numerical answers, show your working or reasoning clearly. Give final answers to a maximum of two significant figures and include appropriate units where applicable.

1.	Explain why it is not possible for you to get up from your seat without first leaning forward. You may use diagrams to illustrate your answer.				[3]

2. 	Explain whether a water glass that tapers down to a narrow base is easier to tip over than one with straight sides. 								[2]

Discuss the difference between a full tapered glass and an empty tapered glass.	[2]

3.	The irregular object show in the figure below hung on a string. Its centre of mass is as shown in the figure.

The object rotates to and fro till it reaches a final position.

a.	Explain why the object rotates to and fro.						[2]

b.	Friction acts to eventually stop the object from rotating. State the position of the centre of mass in the object’s final position.						[1]

4. 	The diagram shows four forces applied to a circular object.	 The radius of the object is r.															[4]

Which of the following describes the resultant force and resultant torque on the object?

Circle correct letter.

|  | resultant force | resultant torque |
| --- | --- | --- |
| A B C D | zero zero non-zero non-zero | zero non-zero zero non-zero |

Justify your answer by calculating the resultant torque (in terms of r) and the resultant force.

5.      A globe is strung up in a classroom as shown in the diagram below.  Note that the roof	and T2 are both horizontal.:

If the mass of the globe is 1.20 kg, determine the tension T1 and T2. The one cable is horizontal to the ground.								[4]

Answer:____________________

6.	A cantilever 5.0 m long is shown below. It has a mass of 800 kg and is held up by two supports X and Y.

Y

X

2.2 m

5.0 m

a.	State the direction of the force exerted by support X on the cantilever.		[1]

b.	Determine the force that each of the supports exerts on the cantilever.		[4]

7. 	Sumo wrestling is a sport with a simple objective: push your opponent so they topple over.  The Sumo wrestler is often a very large person with a lot of weight.  When they begin the wrestlers move their feet apart on the floor and crouch down low to the floor.

a.	Explain why Sumo wrestlers stand in this way.				[2]

b.	A 180 kg Sumo wrestler goes to sit 10 cm from the left end of a 250 cm long bench. The bench has a mass of 4.50 kg.

(i)	How far from the other end of the bench should an 80 kg spectator sit to just stop the bench from tipping if the legs of the bench are 60 cm from each end?  Draw and label all forces and measurements on the diagram below.		[6]

Answer:____________________

(ii)	Each leg of the bench can only withstand a maximum force of 2500N. Will the bench break with the Sumo wrestler and the spectator seated as above? Show your working out.								[3]

Answer:____________________

8.	The diagram below shows a side view of a laptop computer resting on an outdoor table. The mass of the base of the laptop is 1.50 kg and the mass of the screen is 600 g. They are both 22.0 cm long. There is an angle of 70.0° between the horizontal and the screen. The computer is blown over by wind.

Assume that the base and screen both have a uniform mass distribution.

22.0 cm

22.0 cm

70.0°

Wind Force

Calculate the minimum single equivalent wind force on the centre of the screen needed to tip the laptop over. 								[4]

Answer:____________________

9.	A ball is placed on different surfaces as shown in the diagrams below.		[3]

Fig. a				Fig. b				Fig. c

State whether the ball is in a state of stable equilibrium, unstable equilibrium or neutral equilibrium and explain your answer in each case.

a.	Fig. a:

___________________________________________________________________________

b.	Fig. b:

___________________________________________________________________________

c.	Fig. c:

___________________________________________________________________________

10.	A decorative mobile contains three unknown weights A, B and C. The strings and rods have negligible weight, and the rods are to hang horizontally. Find the weights of the balls A, B and C. 									[4]

Answer:____________________

11.	A person climbs a ladder that leans on wall as shown below.

A

The ladder is 3.0 m long and rests at a distance of 1.80 m away from a wall. The man stands with his feet 0.500 m above the ground. The ladder has a negligible mass, and the man has a mass of 75.0 kg.

a.	Determine the angle the ladder makes with the ground.				[1]

Answer:____________________

b.	Calculate the force that the wall exerts on the ladder at position A. Assume that the force acts at right angles to the ladder. 						[6]

Answer:____________________

c.	If the person climbs higher up the ladder, explain qualitatively how the force exerted on the ladder by the wall changes.							[3]

END OF PAPER